DESIGN FILE,,,,

module traffic_3road (

input clk,

input reset,

input emergency,

input ped_req,                // pedestrian request

output reg [2:0] lightA,

output reg [2:0] lightB,

output reg [2:0] lightC,

output reg ped_signal          // pedestrian WALK signal

);

// State declaration

parameter S0 = 3'b000, // A Green

S1 = 3'b001, // A Yellow

S2 = 3'b010, // B Green

S3 = 3'b011, // B Yellow

S4 = 3'b100, // C Green

S5 = 3'b101, // C Yellow

S6 = 3'b110; // Pedestrian Crossing

parameter PED_TIME = 5; // pedestrian crossing time

reg [2:0] state, next_state;

reg [3:0] counter;

// State & counter register

always @(posedge clk or posedge reset) begin

if (reset) begin

state   <= S0;

counter <= 0;

end else begin

state <= next_state;

if (state != next_state)

counter <= 0;

else

counter <= counter + 1;

end

end

// Next state logic

always @(*) begin

next_state = state;

if (emergency)

next_state = S0; // Road A priority

else begin

case (state)

S0: if (counter == 10) next_state = S1;

S1: if (counter == 3)

next_state = ped_req ? S6 : S2;

S2: if (counter == 10) next_state = S3;

S3: if (counter == 3)

next_state = ped_req ? S6 : S4;

S4: if (counter == 10) next_state = S5;

S5: if (counter == 3)

next_state = ped_req ? S6 : S0;

S6: if (counter == PED_TIME)

next_state = S2; // resume traffic

default: next_state = S0;

endcase

end

end

// Output logic (Moore FSM)

always @(*) begin

// default outputs

lightA = 3'b100;

lightB = 3'b100;

lightC = 3'b100;

ped_signal = 1'b0;

case (state)

S0: lightA = 3'b001; // A Green

S1: lightA = 3'b010; // A Yellow

S2: lightB = 3'b001; // B Green

S3: lightB = 3'b010; // B Yellow

S4: lightC = 3'b001; // C Green

S5: lightC = 3'b010; // C Yellow

S6: ped_signal = 1'b1; // Pedestrian WALK

endcase

end

endmodule